# 02 - Technical approach

Status: DRAFT_FOR_OWNER_REVIEW. Bidder: Vyovrinda Aerospace. Tender 2026_DRDO_788433_1, RFP

DTDF/06/13516/DSP/ABEP/X/L/M/01 (Part III). Technical source: commit b0937e8 (b0937e88b4ad21cb31c5778aac4b3dca88fdd94a, pinned by

docs/bid/bid_technical_baseline_v2.json). Every path cited below is a file at that commit (git show b0937e8:<path>).

Nothing in this document is submitted by the repository.

Quantity labels (CLAUDE.md rule 10, `docs/EVIDENCE.md`). Every number carries one label: *measured*, *model-derived*,

*assumed*, *owner-allocation* (an owner design allocation, not evidence), *design target* (an owner target, not evidence)

or *pending* (no value exists yet). There is no *measured* Vyovrinda hardware value at the technical source: no H-1, ICP,

PPU or upstream hardware has been built or tested.

## 1. Validation status (read first)

| item | status at b0937e8 | source |
| --- | --- | --- |
| Architecture | INVESTIGATION_HYPOTHESIS (topology decided and frozen for development; physical design not frozen); frozen reference flight architecture: false | docs/architecture/freeze_candidate/architecture_freeze_candidate_v1.json #/architecture_status |
| Hall-transport validation (gate 3) | FAIL. P5-Xe: transport not identifiable; P5-N2 v1: all 9 screening candidates INCONCLUSIVE / NOT ELIGIBLE (permanent) | CLAUDE.md section "Gate status"; hallthruster_bridge/validation/VALIDATION_RELEASE_v1.json |
| Credible Hall transport set | EMPTY (members = []) | hallthruster_bridge/ensemble/transport_ensemble_v0.json #/members |
| Absolute Hall performance (thrust, Isp, efficiency, discharge current, life) | none offered. Every absolute result of the superseded 0-D Hall closure is WITHDRAWN and is not quoted anywhere in this package | CLAUDE.md section "Superseded / withdrawn (do not quote)" |
| RVM, flight configuration hall_icp_neutralizer | 27 NOT_EVALUATED, 3 INCOMPLETE_EVIDENCE, none PASS | docs/requirements/rvm_a9/rvm_a9_v1.json #/status_counts |
| Architecture-level gates AG-01..AG-15, GNG-ICP-01 | evidence sufficient for freeze: AG-15 only (requirement basis; not a compliance result); not sufficient: AG-01, AG-02, AG-03, AG-04, AG-05, AG-06, AG-07, AG-08, AG-09, AG-10, AG-11, AG-12, AG-13, AG-14, GNG-ICP-01; none is PASS / GO (table in 01_COMPLIANCE_MATRIX.md) | docs/architecture/freeze_candidate/architecture_freeze_candidate_v1.json #/architecture_gates, #/pre_lock1_gates |
| Upstream design (intake / compressor / feed) | robust upstream set EMPTY under the frozen 196-state design-state set (F9-DF-01 EMPTY_ROBUST_SET_NOT_EVALUATED) | docs/architecture/freeze_candidate/architecture_freeze_candidate_v1.json #/design_findings_for_owner[id=F9-DF-01] |
| Mass | MASS INCOMPLETE_EVIDENCE / NOT_YET_CLOSED (section 9) | docs/budgets/mass_power_a9_v4/mass_power_a9_v4.json |

The programme is therefore hardware-first. The owner moved the decisive-thrust question from literature and

simulation to a controlled hardware experiment (docs/decisions/OD_HARDWARE_PIVOT_2026_09_27.json). The order of that

hardware programme is A9.21 (docs/experiments/hall_icp/programme/hw_programme_a9_21_v1.json); see

03_DEVELOPMENT_AND_TEST_PLAN.md. Thrust, power and life numbers will come from measurements on the Vyovrinda hardware,

not from the simulator.

## 2. Architecture

Owner decision A9.19 (docs/decisions/OD_2026_10_01_A9_19_architecture_xe_contingency_owner_decision.json

#/architecture), status of the A9 line OWNER_AUTHORIZED_INVESTIGATION_HYPOTHESIS_NOT_FLIGHT_BASELINE

(docs/decisions/OD_HARDWARE_PIVOT_2026_09_29_A9_hall_downstream_rf_icp_neutralizer.json); flight architecture

hall_icp_neutralizer (docs/architecture/freeze_candidate/architecture_freeze_candidate_v1.json

#/configuration/flight_architecture):

one Hall accelerator (H-1): ionization zone and acceleration zone in one annular channel (RFP Figure 1 "Thruster

(Ionization zone | Acceleration zone)");

one downstream 13.56 MHz RF inductively coupled plasma (ICP) electron source / neutralizer, cathodeless and

electrodeless, serving both supply modes. Topology precedent only: Takahashi et al., J. Electr. Propuls. 3:18 (2024)

(cited in the A9 decision);

two propellant supply modes: AIR_PRIMARY + XE_CONTINGENCY - ambient atmospheric propellant (primary) and xenon (contingency /

emergency; RFP-P17-05 "an extra input system to take care any problems on board unforeseen problems", RFP-P18-08 two

separate tanks; #/xenon_role);

flight cathode: NONE - no conventional hollow cathode and no flight keeper in the flight

architecture. The heated Xe-fed LaB6 hollow cathode C1 is GROUND_REFERENCE_ONLY (owner decisions A9.20 and A9.25

message 8 section 3, docs/decisions/OD_2026_10_04_A9_25_pre_bid_owner_decisions.json #/owner_decision_summary/C1):

it registers I_d,max,H1,Ar on H-1 and is the bench control in the C1-vs-ICP comparison; it is never flight hardware, a

flight fallback, a flight electron source, flight Xe hardware, flight mass, flight power, a flight thermal load or part

of flight architecture closure (RVM-30).

### 2.1 Mapping onto the RFP logical block diagram (RFP-P16-02)

| RFP block (Figure 1) | proposed element | repository lane at b0937e8 | status |
| --- | --- | --- | --- |
| Intake | passive intake (TPMC reduced-order model, Maxwell + CLL wall models) | docs/design_synthesis/f1_intake/f1_intake_synthesis_v1.json | INVESTIGATION_HYPOTHESIS, screening only |
| Filter | filter stage (protection benefit NOT_EVALUATED; a context axis, not searched) | docs/design_synthesis/f2_filter/f2_filter_stage_v1.json | INVESTIGATION_HYPOTHESIS |
| Compressor | compressor (Gaede / turbo-row characteristic) | docs/design_synthesis/f3_compressor/f3_compressor_synthesis_v1.json | INVESTIGATION_HYPOTHESIS |
| Gas Chamber | plenum / feed with set-pressure control | docs/design_synthesis/f4_plenum/f4_plenum_feed_v1.json | INVESTIGATION_HYPOTHESIS |
| Valve | atmospheric metering / isolation valve (driver load slot flow_control_atmospheric) | docs/architecture_comparison/power_boundary_a9_v2/bus_power_boundary_a9_v2.json #/slots | load TBD |
| Xenon Gas -> Valve | Xe storage and flow hardware (AL-08: tank, regulator, two series latch isolation valves + one proportional flow-control valve, plumbing, mounting / thermal), driver slot flow_control_xe | docs/budgets/mass_power_a9_v4/mass_power_a9_v4.json #/lines/hall_icp_neutralizer[line=AL-08] | MEV planning floor 5.9148 kg, PROVISIONAL_PLANNING_FLOOR_NOT_FROZEN |
| Thruster (ionization + acceleration) | H-1 Hall accelerator, magnetically shielded topology T2 with MC-1 electromagnet (H1F-MC-01) | docs/hardware/h1_freeze_candidate/h1_freeze_candidate_v1.json | NOT_FROZEN engineering freeze candidate |
| (added, outside Figure 1) | downstream RF-ICP neutralizer | docs/interfaces/icp_neutralizer/ICP_NEUTRALIZER_ICD.md; docs/design_synthesis/f6_icp_geometry/f6_icp_geometry_v1.json | ICD DRAFT_PENDING_OWNER; geometry search not run |

The ICP neutralizer is an addition to the RFP diagram (the diagram shows no electron source). It replaces the hollow

cathode that a conventional Hall thruster would carry. The owner should decide whether the bid explains this addition

explicitly next to Figure 1 (recommended; no requirement is changed by it).

### 2.2 Subsystem breakdown (RFP-P19-03)

| RFP subsystem | contents | mass lines at the technical source (governing basis) |
| --- | --- | --- |
| a. Air intake and compressor storage | intake / filter / duct; compressor + drive; plenum / feed; Xe storage / flow (separate path) | AL-01 3.5 kg (ALLOCATION_MEV); AL-02 5.5 kg (ALLOCATION_MEV); AL-03 1 kg (ALLOCATION_MEV); AL-08 5.9148 kg (MEV_PLANNING_FLOOR) |
| b. Power supply electronics | Hall PPU incl. collector / bias supply; RF generator / matching; controls / valve drivers / flight sensors | AL-07 6 kg (MEV_PLANNING_FLOOR); AL-06 1.5 kg (ALLOCATION_MEV); AL-09 no value in this record |
| c. Thruster | H-1 head + magnet incl. anode heat-removal hardware; ICP neutralizer incl. collector / bias electrode | AL-04 4.2048 kg (MEV_PLANNING_FLOOR); AL-05 2 kg (ALLOCATION_MEV) |
| (system) | structure / thermal; harness | AL-10 2.5 kg (ALLOCATION_MEV); AL-HAR 1.691 kg (harness rule, computed in the roll-up; not a routed harness) |

Source: docs/budgets/mass_power_a9_v4/mass_power_a9_v4.json #/lines/hall_icp_neutralizer and #/rollups[configuration=hall_icp_neutralizer] at

b0937e8 (values read by the generator). ALLOCATION_MEV = *owner-allocation*; MEV_PLANNING_FLOOR = owner-stated MEV

planning floor (1.20 x an analog / preliminary-design floor). None of these values is a current best estimate (CBE) or

a measured mass.

## 3. Thruster: H-1 Hall accelerator

What exists at b0937e8 is an engineering-article definition, docs/hardware/h1_freeze_candidate/h1_freeze_candidate_v1.json

(status NOT_FROZEN; FREEZE_CANDIDATE items are candidates only):

topology: T2 magnetically shielded, MC-1 electromagnet only, inner + outer coil with trim-coil provision (H1F-MC-01,

-02; *owner-allocation* / *assumed*);

channel: only windows exist, no design point. Annulus area 17.02-35.34 cm^2, mean diameter 45.37-100.6 mm,

width 7.758-17.21 mm, length 66.74-206.5 mm (H1F-CH-02, -04, -05, -10; *model-derived* from xenon scaling rules and

published analogs). The design point (H1F-CH-11) is TBD and will be selected at H1-S7.2 on magnetic feasibility,

thermal margin, mass, packaging and manufacturability, not on thrust (A9.21 item 6);

sizing flow about 1.3 mg/s delivered atmospheric flow, operability / thermal characterization range 0.38-3.2 mg/s

(H1F-CH-12; *owner-allocation*; characterization coverage, not a requirement);

anode: 316L REJECTED_AS_CURRENT_BASELINE; final material OPEN (P4); heat-removal path, allowable and operating

temperatures TBD (H1F-AN-04..-08); electrical rating basis 350 V V_d plus margin TBD (H1F-AN-10);

B(z): no Vyovrinda-specific B(z) evidence exists yet (H1F-BZ-01: no FEMM of MC-1, no measured map); the P5 field

profile is a reference for P5 only.

No thrust, specific impulse, efficiency or discharge current is stated for H-1: no transport closure is admitted

(AG-02 EMPTY) and the measured H-1 thrust / feed map (A9.21 item 11) does not exist yet.

## 4. Electron source / neutralizer: downstream RF-ICP

13.56 MHz RF ICP electron source downstream of the Hall exit (A9 decision). RF matching:

LOCAL_MATCH_SELECTED_FOR_DEVELOPMENT; RF component ratings TBD_AFTER_IMPEDANCE_MAP; ICP RF power closure

PENDING_HARDWARE; ICP electron-current capacity PENDING_ICP45 (A9.2 statuses,

docs/architecture/freeze_candidate/architecture_freeze_candidate_v1.json #/a9_2_statuses).

ICP-45 capacity is defined discharge-OFF as I_e,cap = I_on - I_off (signed) and stays NOT_EVALUATED until

I_d,max,H1 is registered on H-1 (AG-04; RVM-15).

ICP gas feed: primary mode G-REUSE (no dedicated ICP Xe feed booked); G-XE is a declared variant

(docs/architecture/freeze_candidate/architecture_freeze_candidate_v1.json

#/configuration/flight_architecture/icp_feed_gas_baseline; *owner-allocation*).

Bench programme: P1 ICP bench (docs/experiments/hall_icp/p1_icp_bench/p1_icp_bench_v1.json,

ENGINEERING_TEST_PLAN_DRAFT_NOT_SCORE_BEARING) and P2 impedance map (docs/experiments/hall_icp/p2_impedance_map/p2_impedance_prep_v1.json,

PREPARATION_ONLY_NOT_RUN).

Mandatory ICP go / no-go gate GNG-ICP-01 before LOCK-1 (owner decision A9.21 item 4): existence and placement

approved; fail-closed (missing evidence -> NOT_EVALUATED, never GO); no criterion is approved. Status at b0937e8:

NOT_EVALUATED, criteria PENDING_OWNER_ACCEPTANCE (#/pre_lock1_gates[id=GNG-ICP-01]).

## 5. Propellant handling (RFP-P18-08, RFP-P17-03..05)

Ambient-air path (AIR_PRIMARY): intake -> filter -> compressor -> gas chamber / plenum -> metering valve -> H-1. The

intake is sized statewise from air density as a function of solar activity and altitude on the frozen NRLMSIS 2.1

dataset (abep_sim/data/atmosphere_msis21_v1.json, a model input, not compliance evidence) and the frozen

196-state design-state set (abep_sim/data/atmosphere_msis21_orbit_v1_design_states_v2.json, BROAD_ENVELOPE_ALL_INCLINATIONS_ALL_LTAN_NOT_MISSION_ICD).

Upstream status at b0937e8 (model-derived, PARAMETRIC_SENSITIVITY inputs): the coupled upstream screen F7 finds a

nominal all-state delivered-flow frontier of 0.01296 mg/s; 0 Pareto members reach the lower end

(0.38 mg/s) of the characterization coverage at every state

(docs/design_synthesis/f7_f8_optimizer/f7_f8_optimizer_v1.json #/findings[id=F78-02]); the F8 robust upstream set

is EMPTY (0 members; F9-DF-01 EMPTY_ROBUST_SET_NOT_EVALUATED). This is a design finding for the owner, not a demonstrated

requirement failure and not a requirement relaxation. Owner flow-gap order: 1 PERFORMANCE_DERIVED_H1_FEED_REQUIREMENT; 2 CAPTURE_COLLECTION; 3 COMPRESSOR_DOMAIN_PUMPING_FEED_EFFICIENCY; 4 SCHEDULED_SETPOINT. **No robust upstream

(intake / compressor / feed) closure is claimed.**

Xe path (XE_CONTINGENCY / emergency): separate Xe tank, regulator, two series latch isolation valves and one

proportional flow-control valve (owner decision AFI-01-S1: the second latch is the second series flight Xe isolation

valve; the cathode-feed flow-control valve is removed), plumbing -> H-1 (and the ICP only in the declared

G-XE variant). AL-08 = 5.9148 kg is a MEV planning floor, PROVISIONAL_PLANNING_FLOOR_NOT_FROZEN

(plumbing and mounting / thermal TBD; supplier quotations rebase the component floors, RFQ3-GAS rev1

docs/procurement/rfq_a9_v3_gas_rev1/rfq3_gas_rev1.json). No conventional hollow-cathode hardware is in flight AL-08.

The flight Xe load is NOT FROZEN: 2 / 5 / 10 kg are planning / sensitivity cases

(docs/budgets/xe_accounting_a9_v3/xe_accounting_a9_v3.json #/status).

Atomic oxygen: O2-bearing ground tests are labelled NO_ATOMIC_O; atomic-O effects are addressed only through the

separate AO life programme (A9 evidence order). The O / O2 electron-impact chemistry v0 tables are DRAFT and unused

(docs/chemistry/o_o2/v0/channel_status_v0.json #/status).

## 6. Power processing from the bus (< 1.5 kW, RFP-P18-01 / -10)

Boundary and gate (docs/architecture_comparison/power_boundary_a9_v2/bus_power_boundary_a9_v2.json,

PRELIMINARY_DRAFT_FOR_OWNER; v2 differs from v1 only in the configuration taxonomy: single flight configuration):

P_bus is all electrical power crossing the spacecraft-side DC boundary. RFP gate: P_bus,1ms,max < 1500 W, steady

and every start-up step, measured with synchronized channels (>= 100 kSa/s, >= 20 kHz bandwidth) (#/gates_and_allocations;

definition FROZEN_A9_ENGINEERING_DEFINITION).

Owner allocations (not gates, not evidence): 1350 W internal design allocation; 300 W common allocation incl. 50 W

controls / thermal; 1300 W context level only (*owner-allocation*).

Load slots of the flight configuration: Hall discharge; inner / outer / trim magnet supplies; 13.56 MHz RF generator

DC input; matching-network actuators; ICP collector / bias; atmospheric and Xe valve drivers; compressor drive;

thermal control; housekeeping / controls; reserved DC port (#/slots). Every load value is TBD; the ledger is

PARTIAL_BOUNDARY (compressor load TBD) and the gate is NOT_EVALUABLE (AG-10 PARTIAL_BOUNDARY).

Bus architecture: configurable spacecraft-input front end (input voltage TBD, A902-12) feeding a regulated 100 V

internal propulsion bus (#/bus_architecture; *owner-allocation*).

PPU topology options (analogs, docs/hardware/h2/h2_4_ppu_bus/h2_4_ppu_bus_v1.json, PRELIMINARY_DRAFT_FOR_OWNER):

discharge supply class LCC resonant converter (NASA SSEP sub-kW breadboard analog) or ZVS full-bridge (1 kW string PPU

analog); separate current-controlled per-coil magnet supplies; start-up / stop / failure-recovery sequencer in the PPU

controller. H2-4 predates A9.19: its cathode heater / keeper supplies apply only to the ground C1 reference, never to

the flight PPU. A mains-powered laboratory RF generator is GROUND/FACILITY_ONLY and never flight P_bus evidence

(RVM-04).

## 7. Redundancy and single-point failure (RFP-P18-02, RFP-P18-09)

The requirement is accepted. No redundancy design and no FMEA exist at b0937e8 (RVM-19 NOT_EVALUATED: "FMEA /

failure-tree analysis of the selected electronics (none selected)"). The architecture failure trees

(docs/architecture_comparison/failure_tree/failure_trees_v1.json) are architecture-comparison work, not an electronics

FMEA. Proposed approach for the owner to confirm (OIR-TEC-02):

electronics FMEA and single-point-failure list at PDR-1 (Milestone 1), against the PPU block diagram;

redundancy concept (which units are redundant, cold or hot, cross-strapping of the sensors) decided by the owner

with its mass and power cost booked, because mass is already INCOMPLETE_EVIDENCE / NOT_YET_CLOSED (section 9);

FDIR in the PSE software at PDR-2 (Milestone 2, RFP-P20-05).

## 8. Electrical / data interface: MIL-STD-1553B (RFP-P18-12)

Offered: a MIL-STD-1553B remote terminal on the propulsion controller for configuration and high-rate data logging with

the satellite onboard computer and data recorder, discrete lines for thruster operation, and the hardware drivers inside

the propulsion system. No interface design, ICD or test artifact exists in the repository (RVM-20). Verification

path (RVM-20): ICD (inspection), 1553B configuration / data-logging test on a representative bus, discrete-line

operation demonstration. The RT implementation (core / transceiver source, use of the standard's dual A/B bus) is an

owner choice (OIR-TEC-06). The interface ICD is planned as a PDR-2 input.

## 9. Mass (< 40 kg, RFP-P18-11) - disclosed open item

Status: MASS INCOMPLETE_EVIDENCE / NOT_YET_CLOSED. No mass compliance is claimed. AG-11 at b0937e8: INCOMPLETE_EVIDENCE (no CBE; AL-04 / AL-07 / AL-08 below evidence floors); RVM-06 (< 40 kg wet): INCOMPLETE_EVIDENCE.

| level (A9.26 message 2 section 7) | content | label |
| --- | --- | --- |
| Requirement | complete flight wet mass < 40 kg (owner's conservative wet reading of the RFP '< 40kg'; DISC-02 recorded for DRDO clarification) | requirement; not relaxed |
| Proposal design target | nominal dry <= 34.0 kg with a 10 % system margin and the 2 kg Xe planning reference -> 39.4 kg wet | DESIGN TARGET (owner decision A9.26), not evidence; the 2 kg Xe case is a planning reference, not the selected Xe load |
| Current provisional planning / evidence roll-up | dry 40.57 kg after a 20 % system margin; wet 42.57 / 45.57 / 50.57 kg at 2 / 5 / 10 kg Xe | CURRENT PROVISIONAL PLANNING / EVIDENCE FLOOR (owner allocations and MEV planning floors; no CBE, no measured mass) |
| Current status | MASS INCOMPLETE_EVIDENCE / NOT_YET_CLOSED | AG-11 and RVM-06 as above |

Roll-up as recorded (docs/budgets/mass_power_a9_v4/mass_power_a9_v4.json #/rollups[configuration=hall_icp_neutralizer], reading MEV_LEVEL_EVIDENCE_BASED (the single owner reading); every value read from the record):

| term | kg | note |
| --- | --- | --- |
| non-harness lines (known) | 32.1196 | AL-09 has no value in this record and is excluded from the known part |
| harness (rule) | 1.6905 | not a routed harness |
| nominal dry | 33.8101 |  |
| system margin 20 % | 6.7620 | as recorded in the mass record |
| dry (after system margin) | 40.5721 |  |
| + 2 kg Xe (planning / sensitivity case) | 42.57 | DOES_NOT_CLOSE vs < 40 kg (exceeds by 2.57 kg) |
| + 5 kg Xe (planning / sensitivity case) | 45.57 | DOES_NOT_CLOSE vs < 40 kg (exceeds by 5.57 kg) |
| + 10 kg Xe (planning / sensitivity case) | 50.57 | DOES_NOT_CLOSE vs < 40 kg (exceeds by 10.57 kg) |

Lines with a CBE: 0; lines with a measured mass: 0. The flight Xe load is NOT FROZEN: 2 / 5 / 10 kg are planning / sensitivity cases, none is the selected load. Legacy card-closure Xe / MEV values are historical model-regression provenance only and are not quoted.

Mass-closure actions (A9.26 message 2 section 7):

AL-07 current cathodeless PPU CBE / requote

AL-08 quote / design rebase

AL-04 completed Hall-head CBE

AL-09 design-derived CBE

actual routed harness

subsystem integration / structural optimization

No line is reduced to force closure, no qualified hardware is removed for mass alone and the requirement is not relaxed (MQ-10; A9.26 message 2 section 4).

Governing owner decisions: A9.25 message 8 sections 6-8 (docs/decisions/OD_2026_10_04_A9_25_pre_bid_owner_decisions.json

#/owner_decision_summary/bid_mass_wording) and A9.26 message 2 sections 4-7 (A9.26 record: post-source governing decision, not contained in the technical source - see README, 'Technical source').

## 10. Thermal, materials, life

Thermal: ICP_COUPLED_THERMAL UNRESOLVED and ANODE_THERMAL_CLOSURE UNRESOLVED; no heat term evaluated on evidenced

inputs; never reported as a thermal pass (docs/experiments/hall_icp/p3_coupled_thermal/p3_coupled_thermal_v2.json;

RVM-17). The inherited P3 cathode thermal node is NOT_USABLE_FOR_FLIGHT_THERMAL_CLOSURE for the cathodeless

architecture; a successor cathodeless thermal model is post-bid work (AFI-03,

docs/decisions/OD_2026_10_04_A9_25_pre_bid_owner_decisions.json #/owner_decision_summary/AFI-03). Owner

allocation: steady heat into the spacecraft mount <= 50 W (RVM-27; *owner-allocation*, provisional).

Materials: final anode material OPEN; every P4 screen INCOMPLETE_EVIDENCE

(docs/experiments/hall_icp/p4_anode_materials/p4_anode_materials_v1.json). RVM-16 current-architecture reading

(OWNER_CONFIRMED_CURRENT_ARCHITECTURE_READING): no flight keeper exists; AO / O compatibility applies to the actual AO / O-exposed components

(anode, RF-ICP electron-source / neutralizer plasma-facing surfaces, collector / bias electrode, gas-path surfaces,

other plasma-facing parts); graphite is not the current flight baseline for an O / AO-exposed plasma-facing or

electron-source surface until erosion / oxidation coupon evidence supports it (not a universal graphite prohibition);

the frozen RVM-16 requirement basis is unchanged.

Life: design basis >= 26,280 h mission and > 15,000 h cumulative energized operation (RVM-12,

-13; frozen engineering constraints config/constraints/engineering_constraints_v1.json, FROZEN (12/12); operating

scenario mission_scenario_v2 (FROZEN_ENGINEERING_CONFIGURATION, FROZEN)). No admissible life analysis exists (needs an admitted Hall closure or measured wear); AO /

lifetime register v5 defines the wear / endurance segments (docs/experiments/lifetime_ao/ao_lifetime_register_v5.json).

## 11. What this approach does not claim

no thrust, Isp, efficiency, power or life value for the proposed system;

no PASS / GO for any gate; GNG-ICP-01, AG-12, AG-13 and ICP-45 are NOT_EVALUATED;

no frozen physical design or design point; no CBE mass and no mass compliance (MASS INCOMPLETE_EVIDENCE / NOT_YET_CLOSED);

no robust upstream closure (robust upstream set EMPTY, F9-DF-01);

nothing about RFP pages 1-15 and 34-40 (not screened by the repository).